RTOS Master Class by SPRINTS Egypt (Udacity)

Implementation of an EDF Schedular Based on FreeRTOS

By : Michael Samir

Msmb.mail@gmail.com

| ID | Task | Periodicity (ticks) | Execution Time (ms) | Calls in Hyper Period |
| --- | --- | --- | --- | --- |
| 1 | Button_1_Monitor | 50 | 0.019 | 2 |
| 2 | Button_2_Monitor | 50 | 0.0193 | 2 |
| 3 | Periodic Transmitter | 100 | 0.02465 | 1 |
| 4 | Uart_Receive | 20 | 0.02352 | 5 |
| 5 | Load_1_Simulation | 10 | 5 | 10 |
| 6 | Load_2_Simulation | 100 | 12 | 1 |

Hyper Period = LCM(Periodicities) =  LCM(50,50,100,20,10,100).

HyperPeriod = 100

CPU Load =  ( Total Time / HyperPeriod )  * 100

Total time = (0.019*2)  +  (0.0193 * 2)  + (0.02465 * 1) + (0.02352*5) + (5*10)  + (12*1)  = 62.21

CPU Load = 62.21 %

Since D=10 ms and W(10) = 5 ms   so Task # 5 : is schedulable.

For Task 4 :

W(20) = 0.02352 + (20/10) * 5 = 10.02352 ms

Since D = 20 ms and W(20) = 10.02352 ms

W(20) < D    so Task 4 is schedulable.

Make the same for other taskes we got that :

Task #1 , 2 , 3 ,6 are schedulable.

Therefore, our system is schedulable.